HOW TO GUIDE

Particle characterization - BET

Submission dates Monday; Friday. 10:00 - 11:00; 10:00 - 11:00. - Request forms should be sent via email as a word document to alab.chemeng@uct.ac.za, preferably before 10:00 - The request form must be submitted before the samples. Minimum mass - A minimum sample mass of 0.5 g is required. Expected BET SA ( m2/g); Sample mass required (g). < 20; 3-5. ≥ 50; 0.5. Degas temperature and hold time - The sample will be degassed to remove contaminants from the surface and pores, i.e., it will be heated under vacuum. For example: Type of sample; Degas conditions. Zeolites; 350 °C, 3 hours. Other catalysts; 200 °C, 3 hours. Mineral ore; 60 °C, overnight. Type of BET analysis BET 10-point; Adsorption and t-plot; BET and BJH. Estimated turnaround time for analysis Turnaround option; Estimated time; Max number per submission. Normal1; 2-3 weeks; N/A. Fast track2; 5-7 working days; 6 samples. ASAP3; - working days; 3 samples. - Normal turnaround is often shorter than 2 – 3 weeks. It accounts for a queue and/or method development for new samples. A large batch of samples (12 samples) submitted might take longer than 2 – 3 weeks. - Recommended for samples that we have analysed previously. A large batch of samples (more than 6) submitted might take longer than 5 – 7 working days. For biochar samples the maximum number of samples are 3. - ASAP turnaround is only recommended for similar samples that we have analysed previously. We should have an established working method already. We do not recommend biochar samples for this turnaround option.

| SECTION A: TO BE COMPLETED BY CUSTOMER |  |
| --- | --- |
| Customer Details |  |
| Name and Surname: | Cell No: |
| Department: | Research group: |
| Supervisor name: |  |
| Cost Centre: | Fund number: |
| Date request form submitted: | Date sample/s submitted: |

| Instrument - Tristar II 3020 - BET surface area, pore volume and pore size analysis |  |  |  |
| --- | --- | --- | --- |
| Type of analysis, indicate with X |  |  |
| BET 10-point analysis | Adsorption and t-plot analysis | BET and BJH analysis |

| Sample Information |  |  |  |
| --- | --- | --- | --- |
| Macroporous (> 50 nm); Mesoporous (2-50 nm); Microporous (< 2 nm); Non-porous. Type of pores in sample – Indicate with X | Expected BET SA – Indicate with X < 20 m2/g; ≥ 50 m2/g. |
| Type of sample – Indicate with X If other, please specify: Mineral ore; Catalyst; Carbon; Soil; Cement. | Degas temperature and hold time Temperature (°C); Time (h). |
| Sample declaration – Indicate with X |  | YES | NO |
| Does your sample contain anything else (e.g., organic species, acid pre-treatment, etc.)? |  |  |  |
| Is your sample stable at the degas temperature you filled in? |  |  |  |
| Is there any safety information we need to know about the sample/s? |  |  |  |
| Have you declared all the information about your sample/s? |  |  |  |
| Specify anything else you would like to tell us about this request: |  |  |  |

| Turnaround time option |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Number of samples | Indicate turnaround option with X |  |  | Supervisor approval for Fast Track |  |
|  | Normal | Fast Track | ASAP | Indicate turnaround option with X |  |

| SECTION B: TO BE COMPLETED BY ANALYST/S |  |
| --- | --- |
| Start and Completion of Analysis |  |
| Analysis request number: RN2026-B |  |
| Request number assigned by: |  |
| Analyst/s assigned to task: |  |
| Date sample/s submitted: | Date analysis started: |
| Date request form submitted: | Date analysis completed: |
| Date request number sent: | Date report/results sent: |

Notes on Sample Preparation and Analysis

Final degas conditions - Tick to indicateþ Time (h); Temperature °C. File Name/s: Sample name; ANC file; SMP file; Analysis successful?. Observations/opinion: